Małopolska Karta Aglomeracyjna — 
System zarządzania transportem zbiorowym 
w Województwie Małopolskim

część I — Budowa systemu 
zarządzania transportem zbiorowym 
na terenie Województwa Małopolskiego

Specyfikacja sprzętu serwerowego

Historia zmian

| Wersja | Data | Opis | Autor | Uwagi |
| --- | --- | --- | --- | --- |
| 0.1 | 2015-03-17 | Utworzenie | AMG.net |  |
| 0.2 | 2015-11-04 | Weryfikacja | Atos |  |

Spis treści

1.	Wstęp	4

2.	Specyfikacja techniczna komponentów	4

2.1.	Serwery	4

2.2.	Pamięci masowe i macierze	6

2.3.	Urządzenia sieciowe	8

2.4.	Specjalistyczne urządzenia IT	10

2.5.	Załączniki	10

# Wstęp

Niniejszy dokument jest częścią Dokumentacji technicznej. Zawiera techniczną specyfikację sprzętu serwerowego.

Dokument został uzupełniony na podstawie zmian wynikających z prac wdrożeniowych.

# Specyfikacja techniczna komponentów

Specyfikacja techniczna komponentów w 1 serwerowni (druga jest identyczna)

## Serwery

| Producent | HP |
| --- | --- |
| Model zestawu | HP BladeSystem c7000 |
| Ilość zestawów | 2 szafy blade |
| Architektura procesorów | W architekturze x86-64 |
| Model jednostki obliczeniowej | Serwer blade HP ProLiant BL680c |
| Ilość jednostek obliczeniowych w zestawie | 6 serwerów blade, po 3 w każdej szafie |
| Pamięć RAM jednostki obliczeniowej | 64 GB wyposażona w mechanizm ECC |
| Model procesora | Intel® Xeon® Processor E7-4850 (24M Cache, 2.00 GHz, 6.40 GT/s Intel® QPI |
| Ilość procesorów w zestawie | 6 procesorów |
| Łączna ilość rdzeni w zestawie | 60 |
| Dyski | 4 dyski 500GB 6G SAS 7.2K 2.5 w każdym serwerze blade |
| Łączna ilość pamięci RAM w zestawie | 192 GB wyposażona w mechanizm ECC w każdej szafie, 384 w serwerowni |
| Karty FC | W sumie 12 portów, po 6 w szafie, po 2 na serwer blade |
| Porty LAN | 6 kart dwuportowych 10Gb w 2 szafach, po 2 porty na serwer |
| Partycjonowanie | Nie |

| Produkt | Opis | Liczba |
| --- | --- | --- |
| 681840-B21 | HP BLc7000 1PH 2PS 4Fan Trl IC Plat Encl | 4 |
| HA104A4 | HP 4y 4h 24x7 HW Support | 1 |
| HA104A4 7FX | c7000 Enclosure HW Supp | 4 |
| AJ820B | HP B-series 8/12c BladeSystem SAN Switch | 8 |
| H7J34A4 | HP 4yr Foundation Care 24x7 Service | 1 |
| H7J34A4 85J | Brocade 4/12 and 4/24 SAN Switch Supp | 8 |
| 451439-B21 | HP BLc Cisco 1/10GbE 3120X Switch | 16 |
| 378929-B21 | Cisco Enet SX Fiber SFP Module | 32 |
| 588603-B21 | HP 2400W Plat Ht Plg Pwr Supply Kit | 16 |
| 412140-B21 | HP BLc Encl Single Fan Option | 24 |
| QK734A | HP Premier Flex LC/LC OM4 2f 5m Cbl | 32 |
| 456204-B21 | HP BLc7000 DDR2 Encl Mgmt Option | 4 |
| AJ716B | HP 8Gb Short Wave B-Series SFP+ 1 Pack | 16 |
| 643781-B21 | HP BL680c G7 E7-4850 64G Svr | 12 |
| HA104A4 | HP 4y 4h 24x7 HW Support | 1 |
| HA104A4 7XF | BL6xxc Svr Bld HW Support | 12 |
| 507610-B21 | HP 500GB 6G SAS 7.2K 2.5in DP MDL HDD | 24 |
| 613431-B21 | HP BLc NC553m DP FlexFabric Adptr Opt | 12 |
| 451871-B21 | HP BLc QLogic QMH2562 8Gb FC HBA Opt | 12 |
| C6N36AAE | HP Insight Control ML/DL/BL Bundle E-LTU | 12 |
| J9575A | HP 3800-24G-2SFP+ Switch | 4 |
| J9575A ABB | Europe - English localization | 4 |
| J9577A | HP 3800 4-port Stacking Module | 4 |
| J9579A | HP 3800 3m Stacking Cable | 4 |
| J9150A | HP X132 10G SFP+ LC SR Transceiver | 8 |
| U3MF9E | Foundation Care NBD Exchange SVC, HW and SW Support, 4 year | 4 |
| AM867B | HP 8/8 (8)-ports Enabled SAN Switch | 4 |
| AM867B ABB | Europe - English localization | 4 |
| H7J32A4 | HP 4Yr Foundation Care NBD Service | 1 |
| H7J32A4 9LJ | HP B-Series 8/8 and 8/24 Switch Support | 4 |
| AJ716B | HP 8Gb Short Wave B-Series SFP+ 1 Pack | 32 |

## Pamięci masowe i macierze

| Ilość | 1 |
| --- | --- |
| Producent | EMC |
| Model | VNC5100 |
| Specyfikacja techniczna | Min/Max Drives 4/75 Array Enclosure3U Disk Processor Enclosure (15x3.5" or 25x25")Standby Power System1U 1.2KWRaid Options 0/1/10/3/5/6 CPU/Memory per ArrayIntel Xeon 5600/8GBEmbedded IO Ports per Array8FC ports and 4 SAS portsMax Total Ports per Array8 2/4/8 Gb/s FC Max8 6Gb/s SAS Buses2 Max Raw Capacity 225TBMax SAN Hosts: 512Max Number of Pools 10Max Number or LUNs512Max LUN Size 16 TB (Virtual Pool LUN)Protocols supported: Lock Manager (NLM) v 1,3,4ICMP LDAP Kerberos Authentication Ethernet TrunkingMicrosoft DFSiSCSI, FC (FCP SCSI-3), FCoENT LAN Manager (NTLM)NTPSNMP V1-V3SNTPVirtual LAN (IEE 802.1q) Windows, Linux Operating Temperatures: 10-40 C Relative Humidity20%-80% non-condensingAltitude: 2286m 40 C3048m 37C |

| Produkt | Opis | Liczba |
| --- | --- | --- |
| VNX51D159010F | VNX5100 DPE 15 X 3.5 DRV 6 X 900 10K FLD | 1 |
| V3-VS10-900 | 3.5 IN 900GB 10K SAS DISK DRIVE | 9 |
| V13-PWR-7 | 2 C13 PWRCORDS W/ CEE7/7 PLUGS 250V 10A | 2 |
| VNXFCSFPS | ADDITIONAL 8 G FC SFP FOR VNX 51/53 | 1 |
| ESRS-GW-200 | EMC SECURE REMOTE SUPPORT GATEWAY CLIENT | 1 |
| VNX51-KIT | DOCUMENTATION KIT FOR VNX5100 | 1 |
| PSINST-ESRS | ZERO DOLLAR ESRS INSTALL | 1 |
| M-PREHW-001 | PREMIUM HARDWARE SUPPORT | 1 |
| M-PRESW-001 | PREMIUM SOFTWARE SUPPORT | 1 |
| WU-PREHW-001 | PREMIUM HARDWARE SUPPORT - WARR UPG | 1 |
| VNXSPSAS | 2ND OPTIONAL SPS FOR VNX 51/53 | 1 |
| UNIB-V51 | Unisphere Block & VNX OE VNX5100 | 1 |
| C13-PWR-7 | 2 C13 PWRCORDS W/ CEE7/7 PLUGS 250V 10A | 2 |
| 456-100-309 | AVAMAR 1 TB INCR CAPACITY LICENSE | 14 |
| 456-103-950 | Backup and Recovery Manager Avamar | 1 |
| M-PRESWDD-H1 | PREMIUM SOFTWARE SUPPORT (DD) | 1 |
| WU-PREHWB-M00 | Premium Hardware Support-WARR UPG(ANDL) | 1 |
| M-PREHWDP-M1 | PREMIUM HARDWARE SUPPORT | 1 |
| AVMA2400FG4S | AVAMAR G4S M2400 STORAGE NODE FLD INST | 2 |
| C13-PWR-7 | 2 C13 PWRCORDS W/ CEE7/7 PLUGS 250V 10A | 4 |
| DSBRLKT-B | DSB SW GEN RCK KIT -B | 4 |
| DS-300B-8G | DS-300B 8/24P 8G BASE SWITCH | 4 |
| M-PREHW-001 | PREMIUM HARDWARE SUPPORT | 1 |
| WU-PREHW-001 | PREMIUM HARDWARE SUPPORT - WARR UPG |  |

## Urządzenia sieciowe

| Ilość | 1 |
| --- | --- |
| Producent | Cisco |
| Model | ASR1001 System |
| Specyfikacja techniczna | Software compatibility Cisco IOS XE Operating System, which is based on Cisco IOS Software Release 12.2SR (Please consult your Cisco account representative for additional details.) Software protocols Refer to Cisco IOS Software 12.2SR protocol support Connectivity ● Console port (RJ-45 connector) ● Auxiliary port (RJ-45 connector) ● 10/100/1000 Ethernet port (RJ-45 connector) ● Two RJ-48 connectors for BITS input clocks Memory options ● Two 1-GB Double Data Rate 2 (DDR2) mini-DIMMs ● Two 2-GB DDR2 mini-DIMMs ● Upgradable memory from 2-GB to 4-GB DRAM Storage options ● 40-GB HDD (RP1 only) ● 1-GB USB Compact Flash memory Performance ● Scalability up to 1,000,000 IPv4 routes or 500,000 IPv6 routes ● BGP RR Scalability up to 5,000,000 IPv4 routes or 3,000,000 IPv6 routes Reliability and availability ● 1 + 1 redundancy in dual-route-processor configuration ● Support for online insertion and removal (OIR) ● Support for NSF and Stateful Switchover (SSO) ● Support for ISSU MIBs ● RFC 2737 compliant Network management ● Telnet and Secure Shell (SSH) Protocol (command-line interface [CLI]) ● Console port (through the CLI) ● Simple Network Management Protocol (SNMP) ● RFC 2665 LEDs ● PWR - Power Green - All power rails are within specifications ● STAT - Status Green - Cisco IOS Software has booted Yellow - BootROM has successfully loaded Red - System failure or during boot process ● ACTV- Active Green - Active route processor ● STBY - Standby Yellow - Standby route processor ● CRIT - Critical Red - Critical alarm or during boot process ● MAJ - Major Red - Major alarm ● MIN - Minor Amber - Minor alarm ● LINK - Management Ethernet link status Solid green - Link with no activity FLASH green - Link with activity Off - No link ● DISK0 - Internal Compact Flash FLASH Green - Activity indicator Off - No activity ● DISK1 - External Compact Flash FLASH green - Activity indicator Off - No activity ● DISK2 - Internal HDD FLASH green - Activity indicator Off - No activity ● CARRIER - BITS interface Off - Out of service Green - In frame and in service Amber - Fault or loop condition Physical dimensions (H x W x D) 0.92 x 16.7 x 14.19 in. (0.02 x 0.428 x 0.36m) Weight 5.0 lb (2.3 kg) Approvals and compliance Safety ● UL60950-1 and CAN/CSA-C22.2 No. 60950-1-03 Information technology equipment ● AS/NZS 60950-1 ● IEC/EN 60950-1 Information technology equipment ● 73/23/EEC Electromagnetic Emissions Certification ● AS/NZ 3548: 1995 (including AMD I + II) Class B ● EN55022: 1998 Class B ● CISPR 22: 1997 ● EN55022: 1994 (including AMD I + II) ● 47 CFR Part 15: 2000 (FCC) Class B ● VCCI V-3/01.4 Class 2 ● CNS-13438: 1997 Class B ● GR1089: 1997 (including Rev. 1: 1999) |

| Produkt | Opis | Ilość |
| --- | --- | --- |
| ASR1001 | Cisco ASR1001 System Crypto 4 built-in GE Dual P/S | 2 |
| CON-PSRT-CASR1001 | PRTNR SS 8X5XNBD Cisco ASR1001 SystemCrypto | 8 |
| SFP-GE-T | 1000BASE-T SFP (NEBS 3 ESD) | 6 |
| M-ASR1K-1001-4GB | Cisco ASR1001 4GB DRAM | 4 |
| ASR1001-PWR-AC | Cisco ASR1001 AC Power Supply | 4 |
| CAB-ACE-RA | Power Cord Europe Right Angle | 4 |
| SASR1001UK9-39S | Cisco ASR 1001 IOS XE UNIVERSAL | 2 |
| SLASR1-IPB | Cisco ASR 1000 IP BASE License | 2 |
| CON-PSRT-SLASR1IK | PRTNR SS 8X5XNBD Cisco ASR 1000 IP BASE License | 8 |

## Specjalistyczne urządzenia IT

Szafa rack

| Ilość | 1 |
| --- | --- |
| Producent |  |
| Model |  |
| Wysokość | 47U |
| Monitor |  |

## Załączniki

| Nazwa | Opis |
| --- | --- |
| EMC VNX5100/VNX5300 | Macierz dyskowa |
| FortiGate -300C | Firewall sprzętowy |
| HP BladeSystem c7000 | Obudowa Blade |
| Cisco ASR 1001 | Router |
| EMC AVAMAR | System archiwizujący |
| EMC CONNCETRIX DS-300B | System pamięci masowych |